Task-1:

- Power Off: Turn off your computer completely and unplug the power cable.

- Open the Case: Remove the side panel of your desktop (or the bottom cover of your laptop).

- Locate the RAM: Find the long, narrow circuit boards attached to the motherboard.

- Remove the RAM: Push down the small plastic clips at both ends of the RAM slot. The RAM stick will pop up. Pull it out gently.

- Re-insert the RAM: Line up the notch on the RAM stick with the ridge in the slot. Push down firmly on both ends until the side clips snap back into place automatically.

- Close and Test: Put the computer cover back on, plug in the power, and turn the computer on.

Common issues to check for:

- Not fully seated: If you do not hear a click, the RAM is not pushed in all the way and the computer will not turn on.

- Dust in the slot: Dust can cause a bad connection. Blow the slot clean before re-inserting the RAM.

Task-2:

- Identify the Problem: Check for signs like a black screen, strange lines or dots on your monitor, or games crashing.

- Check Cables and Monitor: Make sure the display cable (HDMI or DisplayPort) is plugged into the GPU, not the motherboard. Try a different cable or monitor.

- Check Physical Connections: Turn off the PC. Unplug and push the GPU back into its slot firmly. Make sure all GPU power cables are clicked in tight.

- Reinstall Graphics Drivers: Turn on the PC. Uninstall your old GPU drivers and download fresh drivers from the manufacturer's website (Nvidia or AMD).

- Test and Confirm the Fix: Open a heavy 3D game or benchmark program. Run it for 15-30 minutes to make sure the screen stays clear and the PC does not crash.

Task-3:

When the Drive is Unplugged

- Error Messages: When you turn on the PC, the screen stays black or shows text like:

  - "Reboot and Select proper Boot device"

  - "No Bootable Device Found"

- BIOS Indicator: Inside the BIOS settings, the drive slot (SATA or M.2/NVMe) will show as "Empty" or "None". The drive is missing from the Boot List.

When the Drive is Reconnected

- BIOS Indicator: When you turn on the PC, go into BIOS. The drive name and size (for example, Samsung SSD 1TB) will show up again under the SATA or NVMe storage list.

- Result: The PC boots up normally into Windows or your operating system without any missing drive errors.

Task-4:

First Component to Check: RAM

Reason: Faulty or loose RAM is the most common cause of sudden restarts and Blue Screen crashes. When memory makes an error, the system instantly restarts to prevent data damage.

### Diagnostic Steps for RAM

- Run Windows Memory Diagnostic: Press Windows Key + R, type mdsched.exe, and select Restart now and check for problems.

- Test One Stick at a Time: If you have two RAM sticks, remove one and run the PC. If it works, the removed stick may be bad.

- Change Slots: Move the RAM stick into a different slot to check if the motherboard slot is dirty or damaged.

- Reseat the Sticks: Take out all RAM sticks, wipe the gold pins gently, and push them back in until they click.