# Practical Session

## Practical 1: Identify Your Network Information

### Command:

ipconfig

### Task:

- Find IPv4 Address

- Find Subnet Mask

- Find Default Gateway

### Learning:

Students will understand how a computer gets a network address from a router.

## Practical 2: Detailed Network Information

### Command:

ipconfig /all

### Task:

Identify:

- Computer Name

- MAC Address (Physical Address)

- DHCP Enabled Status

- DNS Server

### Learning:

Students learn how devices are uniquely identified on a network.

## Practical 3: Test Internet Connectivity

### Command:

ping google.com

### Task:

Observe:

- Reply from server

- Time (ms)

- Packets Sent/Received

### Learning:

Understand how to test if the internet connection is working.

## Practical 4: Continuous Ping Monitoring

### Command:

ping -t google.com

### Stop:

Ctrl + C

### Learning:

Monitor network stability in real time.

## Practical 5: View Saved Wi-Fi Networks

### Command:

netsh wlan show profiles

### Task:

List all Wi-Fi networks previously connected to the system.

### Learning:

Understand how Windows stores wireless network profiles.

## Practical 6: Display Connected Network Information

### Command:

netsh wlan show interfaces

### Task:

Find:

- Wi-Fi Name (SSID)

- Signal Strength

- Radio Type

- Connection State

### Learning:

Understand wireless network connection details.

## Practical 7: Check Active Network Connections

### Command:

netstat

### Learning:

See active network sessions and communication.

## Practical 8: View Network Adapter Details

### Command:

getmac

### Task:

Find MAC Address of Ethernet and Wi-Fi adapters.

### Learning:

Understand hardware identification in networking.

# Hands-On Activity 1: Identify Networking Hardware

Provide actual devices and ask students to identify:

- Router

- Switch

- Access Point

- Ethernet Cable

- Fiber Cable (if available)

- RJ45 Connector

### Task:

Write the name and purpose of each device.

# Hands-On Activity 2: Cable Identification Challenge

Show different cables:

- HDMI

- USB

- Ethernet

- VGA

### Task:

Students identify:

- Cable Name

- Purpose

- Network Cable or Not

# Hands-On Activity 3: Build a Basic Network Diagram

Students draw:

Internet

↓

Modem

↓

Router

↓

Switch

↓

PC1   PC2   Printer

### Learning:

Understand network topology.

# Networking Tips & Tricks

### Tip 1: Find Your Computer Name

hostname

### Tip 2: Find Default Gateway Quickly

ipconfig | find "Default Gateway"

### Tip 3: Check DNS Servers

nslookup google.com

### Tip 4: Display Routing Table

route print

Shows how data travels through the network.

### Tip 5: Flush DNS Cache

ipconfig /flushdns

Useful when websites are not opening correctly.

### Tip 6: Renew IP Address

ipconfig /release

ipconfig /renew

Useful for troubleshooting network connectivity issues.

# Mini Troubleshooting Task

### Scenario:

Internet is not working on a PC.

### Student Checklist:

- Check Ethernet cable connection.

- Check router power.

- Run:

- Ipconfig

- Ping:

- ping 127.0.0.1

- Ping:

- ping google.com

- Identify whether the problem is:

  - Cable issue

  - Router issue

  - ISP issue

  - DNS issue