Task-1:

  Faulty RAM (Memory)

- Symptom: The PC crashes often, shows a blue screen, or freezes unexpectedly.

  Overheating Processor (CPU)

- Symptom: The PC suddenly turns off while playing games or running heavy programs.

  Failing Power Supply (PSU)

- Symptom: The computer does not turn on at all, or power cuts off randomly.

  Failing Hard Drive or SSD

- Symptom: The computer runs very slowly, files get corrupted, or it says "No Boot Device Found."

  Bad Graphics Card (GPU)

- Symptom: Strange lines or weird colors appear on the screen, or games freeze immediately.

Task-2:

1.Check the wall outlet and cable connection:

Ensure the plug is firmly in the wall and the charger cable is tight in the laptop port. Try a different power outlet to confirm the wall socket works.

2.Inspect the charger cable and plug:

Look for broken wires, bends, or damage on the cord. Check the connector pin that goes into the laptop to see if it is bent or broken.

3.Check the laptop charging port:

Look inside the power port on the laptop for dust or debris. Clean it gently with a dry toothpick. Make sure the port is not loose or wobbly.

4.Check the charger brick light:

Look for an indicator light on the power adapter. If the light is off while plugged in, the charger itself is likely dead and needs replacement.

5.Perform a hard power reset:

Unplug the charger. If possible, remove the battery. Hold down the power button for 30 seconds to drain leftover power. Put the battery back in, plug it in, and turn it on.

Task-3:

Test Setup & Results

- Target Reading: A normal USB charger gives around 5.00V.

- Measured Voltage: 5.10V

- Status: Within expected range. The safe range for a USB port is 4.75V to 5.25V.

How to Test It Yourself

- Set your multimeter knob to DC Voltage (DCV / V=) on the 20V setting.

- Touch the red probe to the positive pin (outer right slot inside the USB port) and the black probe to the ground pin (outer left slot).

- Read the number on the screen. If it reads between 4.75V and 5.25V, your charger is working correctly.

Task-4:

1.Turn off power and disconnect:

Turn off the power supply switch and unplug it from the wall. Unplug the 24-pin ATX connector from the motherboard.

2.Jump-start the power supply:

To turn on the power supply without a motherboard, insert a paperclip to connect the green wire (PS_ON) to any black wire (Ground). Plug the power cord back into the wall and turn on the power supply switch.

3.Set up the multimeter:

Turn your multimeter dial to DC Voltage (DCV) on the 20V setting.

4.Probe the wires:

Insert the red probe into a slot with a yellow wire (+12V). Insert the black probe into a slot with a black wire (Ground).

Measured Voltage: 11.3V (below normal)

- A normal +12V rail should measure between 11.4V and 12.6V.

- Reading below 11.5V indicates a failing Power Supply Unit (PSU).

- When the computer runs heavy games or programs, the failing PSU cannot supply enough power, causing the PC to shut down to protect itself.