Task-1:

- Check the power outlet and charger light.

What you are checking: Make sure the wall socket has electricity and the light on your charger turns on.

- Inspect the charger cord and port.

What you are checking: Look for broken wires, bent pins, or a loose connection where the charger plugs into the laptop.

- Perform a hard reset.

What you are checking: Unplug the charger and hold the power button down for 30 seconds. This removes trapped static electricity that can block the laptop from starting.

- Test without the battery.

What you are checking: If the battery can be removed, take it out and plug the laptop into the wall. This tests if a damaged battery is stopping the system from turning on.

- Look for lights or fan sounds.

What you are checking: Watch for keyboard lights or listen for fan noise when pressing the power button. This tells you if the laptop is actually on, but has a broken screen.

Task-2:

1.Check power and status lights:

Make sure the monitor is plugged into the wall and turned on. Look for a small light on the monitor frame. If there is no light, check the power cable.

2.Check the video cable:

Ensure the display cable (HDMI, DisplayPort, or VGA) is firmly plugged into both the monitor and the back of the computer. If the computer has a graphics card, make sure the cable connects to the graphics card, not the motherboard.

3.Verify the input source:

Press the Input or Source button on the monitor. Select the correct connection mode (such as HDMI 1, HDMI 2, or DisplayPort) to match your cable.

4.Test display settings:

If using Windows, press Windows Key + P. Press the Down Arrow and Enter to switch display modes (in case the screen is set to extend to a missing secondary display).

5.Test with another cable or monitor:

Try using a different video cable or plug the monitor into another computer. This confirms if the issue is a dead monitor, a broken cable, or a computer hardware fault.

Task-3:

1.Test the mouse hardware:

Turn the mouse off and on. Check if the battery is dead and put in a fresh one. If it uses a USB receiver, plug it into another computer. If it still does not work, the mouse itself is broken.

2.Test the USB port:

Unplug the mouse USB receiver and plug it into a different USB port on the same computer. Since the keyboard works, plug the mouse into the exact port the keyboard is using. If the mouse works there, the original USB port is damaged.

3.Check system settings and drivers:

Use the keyboard to press Windows Key + X and open Device Manager. Check Mice and other pointing devices to see if the mouse is listed or shows an error icon. Try reinstalling the mouse driver or checking if Bluetooth is turned on in settings.

Task-4:

  Dirty or blocked fans and vents

- What to check: Open the computer case and look for heavy dust buildup on the fans and air vents. Dust stops fresh air from cooling the internal parts.

  Broken cooling fans

- What to check: Turn the computer on and watch if all cooling fans (especially the CPU fan and graphics card fans) are spinning quickly and smoothly.

  Dry or old thermal paste

- What to check: Remove the CPU cooler and check the grey paste between the processor and heat sink. If it is dry, cracked, or missing, it cannot transfer heat away from the processor.