Task-1:

### Key Differences

- Material: Ethernet cables use copper wire (electricity), while fiber optic cables use glass or plastic threads (light).

- Speed & Distance: Fiber optic is much faster and can send data over long distances without losing signal, whereas Ethernet slows down over long distances.

- Durability & Interference: Ethernet cables can suffer from electrical interference; fiber optic cables are completely immune to it.

### Real-World Examples

- Ethernet Cable: Connecting a gaming PC or smart TV directly to your home Wi-Fi router.

- Fiber Optic Cable: Connecting high-speed internet between major cities or undersea data lines connecting countries.

Task-2:

Task-3:

### Steps to Crimp the Cable:

- Strip the Outer Jacket

  - Use a cable stripper to strip off about 1.5 inches (3–4 cm) of the outer plastic jacket from the end of the cable. Be careful not to nick the internal copper wires.

- Remove the Center Plastic Core (Spline)

  - CAT6 cables usually have a plastic separator (spline) in the middle. Cut this plastic core as close to the jacket base as possible using small wire cutters or scissors.

- Untwist and Straighten the Wires

  - Untwist all 4 pairs of colored wires completely and flatten them out straight using your fingers so they are easy to arrange.

- Arrange Wires in T568B Order

  - Line up the wires side-by-side in this exact order from left to right:

    - White-Orange

    - Orange

    - White-Green

    - Blue

    - White-Blue

    - Green

    - White-Brown

    - Brown

- Trim the Wires Straight

  - Pinch the arranged wires tightly together. Cut them completely straight across, leaving about half an inch (1.2 cm) exposed from the outer jacket edge.

- Insert Wires into the RJ45 Connector

  - Hold the RJ45 connector with the plastic release clip facing down (pins facing up).

  - Push the wires firmly into the connector until all 8 copper ends reach the very tip of the plug. Make sure the outer jacket is pushed slightly inside the back of the plug for extra strain relief.

- Crimp the Plug

  - Insert the loaded RJ45 connector into the matching slot on your crimping tool.

  - Squeeze the handles firmly together until you feel the tool lock and press the metallic pins down into the wires.

Task-4:

For a 10-PC LAN gaming cafe, Ethernet cables and an RJ45 crimper are the best choice. Since all computers are close together in one room, Ethernet provides plenty of speed at a much lower cost and is easy to set up. Fiber optics and expensive tools like fiber cleavers are unnecessary for short distances and would unnecessarily raise setup costs.